Assignment 2 Answers and Explanation

Task2

Suitable IP range for HQ and branch

HQ supports up to 8000 devices and branch supports up to 500 devices.

Private block of 10.10.0.0/16 (IPv4 address)

An IPv4 has 32 bits, so to find host bits:

32 bits - 16 network bits = 16 host bits

To calculate how many usable IP addresses a subnet provides:

Usable hosts = 2^h - 2

2^13 - 2 = 8190 (supports more than 8000 devices for HQ)

2^9 - 2 = 510 (supports more than 500 devices for branch)

To find network bits, subtract from 32 bits:

32 bits - 13 host bits = /19 (HQ)

32 bits - 9 host bits = /23 (branch)

IPv4 has 4 octets (32 bits) so each octet = 8 bits:

/19 takes up 2 octets + 3 bits of 3rd octet = 224

/23 takes up 2 octets + 7 bits of 3rd octet = 254

Hence subnet masks are:

/19 = 255.255.224.0

/23 = 255.255.254.0

To calculate block size:

256 - 224 = 32

256 - 254 = 2

|  | HQ Range | Branch Range |
| --- | --- | --- |
| Network | 10.10.0.0 | 10.10.32.0 |
| Usable Range | 10.10.0.1 - 10.10.31.254 | 10.10.32.1 - 10.10.33.254 |
| Broadcast | 10.10.31.255 | 10.10.33.255 |

Network Diagram

RouterHQ Routing Table

| Destination | Gateway | Genmask | Flags | Iface |
| --- | --- | --- | --- | --- |
| 10.10.0.0 | * | 255.255.224.0 | U | eth1 |
| 10.10.32.0 | 10.10.0.2 | 255.255.254.0 | UG | eth1 |
| 172.20.1.0 | * | 255.255.255.0 | U | eth0 |
| default | 172.20.1.1 | 0.0.0.0 | UG | eth0 |